# Enterprise Hospitality IPTV Platform — Revised Production Architecture

## Executive Summary

This project proposes a production-grade hospitality IPTV and guest experience platform for a 151-room luxury hotel deployment using native LG Hospitality webOS commercial televisions.

The revised architecture fundamentally shifts from a simple “TV web application” model into a resilient distributed device platform designed for:

- high operational reliability

- automated recovery

- secure guest isolation

- centralized orchestration

- enterprise observability

- fault-tolerant state synchronization

The solution replaces fragile hospitality hardware stacks involving:

- external Android TV boxes

- IR blasters

- HDMI splitters

- multiple remotes

- unmanaged consumer interfaces

with a software-defined architecture running directly on LG commercial webOS televisions.

The platform integrates with Oracle Opera PMS and enables:

- automated check-in personalization

- seamless satellite TV control

- native OTT launching

- centralized room orchestration

- automated checkout privacy purge

- multi-TV suite synchronization

- remote device monitoring

- self-healing TV runtime recovery

The revised architecture prioritizes:

- State consistency

- Recovery engineering

- Device orchestration

- Hardware reliability

- Operational scalability

rather than only frontend application development.

# I. Revised System Philosophy

## 1. Original Problem

Traditional hospitality IPTV deployments fail due to excessive hardware dependency.

Typical failures include:

- IR blaster desynchronization

- Android box crashes

- HDMI splitter instability

- multiple remote confusion

- firmware inconsistency

- operational maintenance overhead

- guest support escalation

The original architecture successfully eliminated much of this hardware complexity.

However, the revised architecture further addresses the hidden distributed systems failures that emerge in real-world hospitality environments.

## 2. Revised Engineering Philosophy

The televisions are no longer treated as “screens.”

Instead, every television is treated as a:

- managed edge device

- authenticated network node

- state-aware runtime

- recoverable client agent

- monitored endpoint

This changes the platform from:

TV Web Application

into:

Distributed Hospitality Device Platform

# II. Revised Core Architecture

## 1. High-Level System Architecture

┌──────────────────────┐
                │   Oracle Opera PMS   │
                └──────────┬───────────┘
                           │
                  (FIAS / OHIP Events)
                           │
                           ▼
             ┌──────────────────────────┐
             │ Event Ingestion Layer    │
             │ (Validation + Parsing)   │
             └──────────┬───────────────┘
                        │
                        ▼
             ┌──────────────────────────┐
             │ Durable Event Queue      │
             │ (Redis Streams/Kafka)    │
             └──────────┬───────────────┘
                        │
                        ▼
             ┌──────────────────────────┐
             │ Central State Engine     │
             │ (Authoritative Logic)    │
             └──────────┬───────────────┘
                        │
         ┌──────────────┴──────────────┐
         ▼                             ▼
┌─────────────────┐        ┌────────────────────┐
│ PostgreSQL      │        │ Redis              │
│ Persistent Truth│        │ Transient Cache    │
└─────────────────┘        └────────────────────┘
         │                             │
         └──────────────┬──────────────┘
                        ▼
            ┌──────────────────────────┐
            │ Device Session Manager   │
            └──────────┬───────────────┘
                       │
                       ▼
            ┌──────────────────────────┐
            │ WebSocket Gateway        │
            └──────────┬───────────────┘
                       │
                       ▼
        ┌────────────────────────────────┐
        │ LG Hospitality TVs             │
        │ Managed Device Runtime Agents  │
        └────────────────────────────────┘

# III. Revised Technical Stack

## 1. Frontend Runtime

### Primary Stack

- React.js

- Enact JS

- TypeScript

### Responsibilities

- spatial D-pad navigation

- guest UI rendering

- OTT launch interface

- channel navigation

- room personalization

- UI recovery mechanisms

### Important Architectural Constraint

The frontend is NOT treated as business logic authority.

The TV runtime acts only as:

- state renderer

- device controller

- edge client

All authoritative logic remains server-side.

## 2. Device Runtime Layer (NEW)

A dedicated device runtime layer is introduced inside the TV application.

This layer is responsible for:

- WebSocket lifecycle management

- heartbeat transmission

- command queue execution

- reconnection logic

- recovery reconciliation

- device health reporting

- watchdog reloads

- local crash recovery

- hardware API abstraction

This layer separates:

UI concerns

from:

device orchestration concerns

which significantly improves long-term reliability.

## 3. Backend Platform

### Recommended Stack

#### Device Orchestration Layer

- Go (Golang)

Reason:

- superior concurrency handling

- low memory overhead

- stable WebSocket scaling

- excellent long-running process reliability

#### Business Logic APIs

- NestJS

Reason:

- structured enterprise APIs

- modular architecture

- maintainable service layers

- excellent TypeScript ecosystem

## 4. State Infrastructure

### PostgreSQL (Authoritative Truth)

Stores:

- room state history

- device mapping

- guest session records

- audit logs

- provisioning metadata

- firmware records

- recovery snapshots

PostgreSQL becomes:

single source of truth

for the platform.

### Redis (Transient Runtime Layer)

Stores:

- active WebSocket sessions

- temporary room cache

- heartbeat states

- transient orchestration state

- rate limiting

- distributed locking

Redis is explicitly NOT treated as permanent state storage.

## 5. Event Queue Layer (NEW)

### Recommended Options

- Redis Streams (minimum)

- Kafka (ideal enterprise deployment)

- NATS (lightweight alternative)

### Purpose

Prevents PMS packet loss.

All incoming PMS events are first persisted into a durable queue BEFORE processing.

This solves:

- duplicate events

- server crash during processing

- ordering issues

- retry safety

- event replay

# IV. Revised Device Architecture

## 1. Device Identity Model

Every television receives:

- immutable device ID

- room mapping

- role designation

- provisioning profile

Example:

{
  "deviceId": "lg-tv-202-bedroom",
  "room": "202",
  "role": "bedroom",
  "firmware": "webos22-v3.1"
}

This avoids dependency on room number alone.

## 2. Device Heartbeat System (NEW)

Every television transmits periodic heartbeat packets.

Example:

{
  "deviceId": "lg-tv-202-main",
  "status": "alive",
  "memoryUsage": "214MB",
  "uptime": "12h",
  "appVersion": "2.4.1"
}

Heartbeat interval:

- every 30 seconds

The backend tracks:

- online/offline status

- stale devices

- unhealthy memory usage

- app crash patterns

- reconnection storms

## 3. Device Watchdog Recovery (NEW)

A watchdog subsystem is embedded into the TV runtime.

### Responsibilities

If:

- UI freezes

- WebSocket disconnects

- runtime hangs

- memory spikes

- rendering stalls

then:

window.location.reload()

is triggered automatically.

The system prioritizes:

automatic recovery

over:

perfect uptime

# V. Revised Network Architecture

## 1. Dedicated IPTV VLAN

VLAN 200 - IPTV Management Network

Strictly isolated from:

- guest Wi-Fi

- public subnets

- hotel staff LAN

This prevents:

- lateral movement attacks

- unauthorized TV access

- device scanning

- malicious traffic injection

## 2. Static IP Architecture

Every television receives:

- static IP assignment

- immutable mapping

- predictable provisioning

Example:

10.20.0.101 → Room 101 Main TV
10.20.0.102 → Room 101 Bedroom TV

## 3. Hardwired Infrastructure

All TVs operate exclusively over:

- Gigabit Ethernet

- Cat6 cabling

Wi-Fi is prohibited for production TV communication.

Reason:

- latency stability

- packet reliability

- multicast stability

- reduced interference

# VI. Revised PMS Event Architecture

## 1. Event Ingestion Flow

Original architecture directly processed PMS packets.

This creates risk.

The revised flow:

Opera PMS
   ↓
Event Queue
   ↓
State Engine
   ↓
TV Synchronization

## 2. Event Object Structure

Every event receives:

- unique event ID

- timestamp

- retry metadata

- processing state

Example:

{
  "eventId": "evt_991122",
  "room": "202",
  "type": "CHECKIN",
  "timestamp": 171600000,
  "processed": false
}

## 3. Idempotent Processing (CRITICAL)

The system must safely handle duplicate PMS packets.

Example:

CHECKIN room 202
CHECKIN room 202
CHECKIN room 202

must still produce:

ONE correct room state

This is enforced through:

- event IDs

- deduplication

- transactional processing

# VII. Revised State Synchronization Model

## 1. Previous Weakness

Original architecture depended heavily on:

push-only WebSocket events

This is dangerous.

If a TV disconnects:

- critical commands may be missed

- checkout purge may fail

- stale guest data may remain

## 2. Revised Synchronization Model

The new architecture combines:

### A. Push-Based Updates

for:

- real-time responsiveness

### B. Pull-Based Reconciliation

for:

- correctness recovery

## 3. Recovery Reconciliation Flow

When a TV reconnects:

TV → “What is my latest authoritative room state?”

Server responds:

{
  "room": "202",
  "guest": null,
  "mode": "vacant",
  "purgeRequired": true
}

This guarantees:

- eventual consistency

- state correction

- missed event recovery

# VIII. HDMI-CEC Reliability Engineering

## 1. Architectural Risk

HDMI-CEC implementations vary heavily between:

- TV firmware versions

- set-top box vendors

- HDMI chipsets

CEC cannot be treated as perfectly reliable.

## 2. Revised Reliability Model

CEC is treated as:

best-effort hardware transport layer

not:

guaranteed command execution layer

## 3. Command Verification Flow

Every CEC command requires:

- transmission

- timeout monitoring

- retry logic

- optional state verification

Example:

Guest selects HBO
    ↓
CEC command transmitted
    ↓
Verification wait
    ↓
Retry if no response

## 4. Firmware Governance (NEW)

Automatic firmware updates are prohibited.

All televisions must:

- run pinned firmware versions

- undergo staged update testing

- validate HDMI-CEC compatibility before rollout

# IX. Multi-TV Suite Synchronization

## 1. Revised Device Mapping

{
  "room_number": "202",
  "guest_name": "John Doe",
  "devices": [
    {
      "deviceId": "tv-202-living",
      "ip_address": "10.20.0.51",
      "role": "living-room",
      "primary": true
    },
    {
      "deviceId": "tv-202-bedroom",
      "ip_address": "10.20.0.52",
      "role": "bedroom",
      "primary": false
    }
  ]
}

## 2. Synchronization Logic

Shared synchronization:

- guest identity

- room state

- checkout purge

- personalization

Independent control:

- HDMI source

- live TV channel

- OTT sessions

- playback state

This prevents:

- cross-TV viewing conflicts

- accidental input hijacking

# X. OTT Application Architecture

## 1. Native OTT Launching

The system launches official native applications.

Example:

luna://com.webos.applicationManager/launch({
  id: "netflix"
})

Advantages:

- hardware acceleration

- official DRM support

- native decoder performance

- 4K playback stability

## 2. OTT Privacy Isolation

At checkout:

- local storage cleared

- cookies removed

- session tokens invalidated

- cached guest data wiped

Additional recommendation:

Force OTT logout verification workflows.

# XI. Provisioning & Deployment Architecture

## 1. Original Problem

Manually configuring 151 TVs is operationally unsustainable.

## 2. Revised Provisioning Model

Every TV performs:

GET /provision

during first boot.

The TV sends:

- MAC address

- firmware version

- serial number

- device ID

Server responds:

{
  "room": "202",
  "role": "bedroom",
  "bootUrl": "/hotel-ui",
  "heartbeatInterval": 30
}

This enables:

- zero-touch onboarding

- remote reprovisioning

- scalable deployment

# XII. Observability & Monitoring (NEW)

## 1. Central Operations Dashboard

A centralized monitoring platform is mandatory.

Dashboard displays:

Room 202:
- TV online/offline
- current guest
- app version
- memory usage
- HDMI health
- last heartbeat
- uptime
- websocket state

## 2. Recommended Monitoring Stack

### Metrics

- Prometheus

### Visualization

- Grafana

### Logs

- Loki

### Error Tracking

- Sentry

## 3. Operational Alerts

Automatic alerts generated for:

- heartbeat timeout

- repeated app crashes

- memory spikes

- CEC failures

- firmware mismatch

- stale room state

- WebSocket storms

# XIII. Security Architecture

## 1. Network Isolation

Strict VLAN segmentation.

## 2. Device Authentication

Every TV authenticates using:

- signed tokens

- device certificates

- provisioning identity

## 3. API Security

Internal APIs protected using:

- JWT

- mTLS internally where applicable

- rate limiting

- service authentication

## 4. Guest Privacy Protection

Critical privacy controls:

- automated checkout purge

- token invalidation

- local cache clearing

- session expiration

- audit logging

# XIV. Developer Sandbox & Testing Strategy

## 1. Simulator Development

Developers use:

- webOS TV Simulator

- webOS SDK

- VS Code extension

Allows:

- UI testing

- WebSocket validation

- D-pad navigation testing

- layout verification

## 2. Hardware Validation Layer

Simulator limitations:

- cannot fully emulate HDMI-CEC

- cannot test DRM playback

- cannot validate hardware APIs

- cannot simulate firmware instability

Therefore:

A physical LG hospitality TV test lab is mandatory.

## 3. Recommended Test Stages

### Stage 1 — UI Sandbox

- React/Enact

- navigation

- layout

- rendering

### Stage 2 — Backend Simulation

- PMS event replay

- Redis sync

- WebSocket recovery

### Stage 3 — Hardware Integration

- HDMI-CEC

- OTT launching

- Luna APIs

- power recovery

### Stage 4 — Soak Testing

Continuous:

- 7–14 day uptime tests

- reconnect storms

- repeated reboot cycles

- firmware recovery testing

# XV. Recommended Production Rollout Strategy

## Phase 1 — Core Reliability MVP

ONLY validate:

PMS Event
   ↓
Reliable State Sync
   ↓
TV Recovery Reconciliation

before implementing advanced UI.

## Phase 2 — Device Runtime Stabilization

Implement:

- heartbeat

- watchdog

- reconnect logic

- provisioning

- monitoring

## Phase 3 — Hardware Integration

Validate:

- HDMI-CEC

- Tata Play interoperability

- firmware stability

## Phase 4 — OTT & Luxury UX

Finally implement:

- polished UI

- animations

- concierge features

- dynamic services

# XVI. Final Architectural Conclusions

This platform is no longer merely:

a smart TV application

It is now architected as:

an enterprise distributed hospitality operating platform

The revised architecture solves the major operational risks involving:

- state inconsistency

- device crashes

- missed checkout purges

- reconnect failures

- firmware instability

- large-scale provisioning

- observability blind spots

The highest-priority engineering goals are now:

- Authoritative state consistency

- Automated recovery

- Device orchestration

- Hardware interoperability validation

- Operational monitoring

rather than frontend complexity.

When properly implemented, the system becomes:

- operationally scalable

- enterprise maintainable

- privacy compliant

- hardware resilient

- deployment friendly

- commercially viable for luxury hospitality environments.

XVII. Detailed Failure Recovery Architecture

This section defines exactly how the system survives real-world failures.

The major architectural shift is:

Failures are assumed normal.
Recovery is engineered intentionally.

1. TV Recovery Lifecycle

Every TV operates using a controlled runtime lifecycle.

Boot Sequence

Power Applied
    ↓
TV Boots
    ↓
Device Runtime Starts
    ↓
Authenticate with Backend
    ↓
Fetch Provisioning Profile
    ↓
Open WebSocket
    ↓
Request Authoritative State
    ↓
Render UI

The important addition is:

Request Authoritative State

This prevents stale sessions after reconnect.

2. Reconnect Recovery Flow

Problem

TV disconnects during checkout purge.

Without reconciliation:

- guest tokens remain

- stale state persists

Revised Recovery Flow

TV reconnects
    ↓
Device runtime requests latest room state
    ↓
Backend compares authoritative state
    ↓
If mismatch detected:
    ↓
Force purge + state correction

Example:

{
  "room": "202",
  "stateVersion": 44,
  "guest": null,
  "purgeRequired": true
}

3. State Versioning (IMPORTANT)

Every room state gets a version number.

Example:

{
  "room": "202",
  "version": 88
}

TV stores last known version.

On reconnect:

TV version = 86
Backend version = 88

Backend knows:

- TV missed 2 events

Then:

- full resync triggered

This is a critical distributed systems pattern.

4. Event Replay Architecture

Since events are durably queued:

the backend can replay missed events.

Example:

CHECKIN
WELCOME_UPDATE
PURGE

can all be replayed safely after outage.

This dramatically improves resilience.

XVIII. Device Session Manager Design

This becomes the central orchestration brain.

1. Responsibilities

The Device Session Manager handles:

- active WebSocket connections

- room-device mapping

- heartbeat tracking

- reconnect handling

- stale session cleanup

- push command delivery

- device health evaluation

2. Session Object Structure

Example:

{
  "deviceId": "tv-202-main",
  "room": "202",
  "connected": true,
  "lastHeartbeat": 171600000,
  "appVersion": "2.4.1",
  "firmware": "webos22-v3.1",
  "stateVersion": 44
}

This becomes operationally critical.

3. Stale Session Cleanup

If heartbeat missing:

No heartbeat > 90 seconds

Then:

mark device offline

This prevents:

- ghost devices

- stale socket routing

- memory leaks

XIX. Enterprise WebSocket Architecture

WebSockets become mission-critical infrastructure.

1. Current Weakness in Most Systems

Most systems:

new WebSocket(url)

and stop there.

This fails under:

- reconnect storms

- packet floods

- network instability

- switch failovers

2. Production WebSocket Requirements

A. Heartbeat Ping/Pong

Backend periodically verifies:

Is connection alive?

B. Exponential Reconnect

Avoid:

151 TVs reconnect simultaneously

after switch reboot.

Use:

1s
2s
4s
8s
16s

backoff.

This prevents backend overload.

C. Session Resumption

On reconnect:

TV sends:

{
  "deviceId": "tv-202-main",
  "lastKnownStateVersion": 44
}

Backend determines:

- incremental sync
OR

- full resync

XX. Database Architecture

1. PostgreSQL Schema Recommendations

A. Devices Table

devices
---------
device_id
room_number
role
firmware_version
last_seen
status

B. Room State Table

room_state
------------
room_number
guest_name
occupancy_status
state_version
updated_at

C. Event Log Table

event_log
------------
event_id
event_type
room_number
payload
processed
timestamp

D. Audit Table

Critical for hospitality compliance.

Tracks:

- purges

- checkouts

- OTT cleanup

- device failures

2. Why Audit Logs Matter

Hotels require accountability.

Example:

Guest claims previous Netflix account visible.

Audit logs prove:

- purge executed

- timestamp

- device acknowledgement

Very important operationally.

XXI. Scalability Engineering

1. Current Scale

151 rooms is moderate scale.

But architecture should support:

1000+ rooms
multi-property deployments

future expansion.

2. Horizontal Scaling

Backend services should remain stateless.

This allows:

Load Balancer
    ↓
Multiple Backend Nodes

Example:

ws-node-1
ws-node-2
ws-node-3

Redis coordinates shared transient state.

3. Why Stateless Matters

If one backend crashes:

another node immediately continues operation.

Without stateless design:

- reconnect chaos occurs

XXII. Advanced Monitoring & Telemetry

This becomes essential at scale.

1. Metrics That Actually Matter

Not just CPU usage.

Real operational metrics:

Device Metrics

- heartbeat latency

- reconnect frequency

- uptime

- memory usage

Network Metrics

- packet loss

- WebSocket disconnect rate

- switch health

Guest Experience Metrics

- average boot time

- channel switch latency

- OTT launch time

- crash frequency

2. Error Classification System

Not all failures equal.

Example:

Severity Levels

P1 — Critical

- checkout purge failure

- PMS desync

- widespread outage

P2 — Major

- HDMI-CEC failure

- repeated TV crashes

P3 — Minor

- delayed heartbeat

- temporary reconnect

This helps operations prioritize correctly.

XXIII. Security Hardening Recommendations

1. Network Security

VLAN ACLs

TV VLAN should ONLY access:

- middleware APIs

- DNS

- NTP

Nothing else.

2. Device Authentication

Every TV should authenticate using:

- signed JWT

- rotating tokens

- provisioning secret

Never trust IP alone.

3. Internal API Security

Use:

- mTLS internally

- service-to-service authentication

- rate limiting

4. Firmware Protection

Disable:

- consumer settings access

- unauthorized app installs

- USB debugging

- external casting

XXIV. Operational Maintenance Strategy

This is where many systems collapse after deployment.

1. Controlled Rollouts

Never update all TVs simultaneously.

Use:

Canary Deployment

Example:

5 TVs
   ↓
20 TVs
   ↓
1 floor
   ↓
entire hotel

2. Remote App Updates

Need centralized deployment pipeline.

Example:

CI/CD Pipeline
    ↓
Versioned TV Builds
    ↓
Controlled OTA Rollout

3. Rollback System

Critical requirement.

If update breaks:

Rollback to previous stable version

immediately.

XXV. Suggested Production Infrastructure

Backend Infrastructure

Core Services

- Go Device Orchestrator

- NestJS API Layer

- Redis Cluster

- PostgreSQL HA

- Kafka / Redis Streams

Monitoring Stack

- Prometheus

- Grafana

- Loki

- Sentry

Deployment

- Docker

- Kubernetes (optional at current scale)

- Nginx ingress

- HAProxy for WebSockets

XXVI. Most Important Engineering Principles

1. State Is More Important Than UI

UI failures are visible.

State failures are dangerous.

2. Recovery Matters More Than Perfection

Systems WILL fail.

Reliable systems:

- recover automatically

3. Hardware Is the Largest Unknown

Software easier to fix than:

- HDMI firmware

- CEC inconsistencies

- chipset behavior

4. Observability Is Mandatory

Without telemetry:

151 rooms = operational blindness

5. Distributed Systems Rules Apply

Even though this is “hotel TV software.”

You are still building:

- distributed state synchronization

- real-time orchestration

- device recovery systems

This is fundamentally distributed systems engineering.

XXVII. Final Technical Assessment

This project is no longer:

a frontend IPTV interface

It has evolved into:

an enterprise distributed hospitality platform

with components involving:

- distributed systems

- embedded runtimes

- networking

- device orchestration

- real-time communication

- hardware integration

- observability engineering

- operational reliability

The strongest aspect of the revised architecture is:

state reconciliation + recovery engineering

because that solves the hidden failures that destroy real deployments.

If I had to break this final architecture in 24 hours, where would it fail first — and why?

Most likely first failure:

Long-duration hardware behavior under real hotel conditions.

Specifically:

- HDMI-CEC instability

- firmware memory leaks

- prolonged uptime degradation

- edge-case recovery bugs after repeated reconnect cycles

Why?

Because software architecture can eliminate most logical failures through:

- queues

- reconciliation

- retries

- observability

- state authority

But hardware behavior under:

- 24/7 uptime

- hotel guest misuse

- power instability

- firmware inconsistencies

remains the hardest real-world uncertainty.